STM32-Cube IDE initial Start-Up

- Open:

If this is the first time you’ve opened STM32-Cube IDE, you should be prompted to select a directory to store all your files.  This is called your workspace.  On RIT computers, this workspace should be created in your student account or on a flash drive.  Do not use the local computer, your work will be lost when the computer reboots. When you install STM32-Cube IDE on your personal computer, you may create this workspace anywhere you like.

If you checked “Use this as the default and do not ask again”, the STM32-Cube IDE launcher window will not appear in the future.

Creating a new Project in STM-Cube IDE

- Open:

- Select:  File  New  STM32 Project, this will invoke the STM32-Cube MX utility

- In the Target Selection window, select Board Selector, the 32F411EDISCOVERY board (fifth one down), and then Next.

- In the Project Setup window, enter a Project Name, select C++, and then Next.

Make sure the location is the workspace you entered previously. If it isn’t, correct the location.

- In the Firmware Library Package Setup window, use all the default settings and then select Finish.

- Select Yes, to initialize all peripherals with their default Mode.

- Select “Remember my decision” and then select Yes.  (note: this window will only appear the first time you run the software).

- Once the project has been created, the Device Configuration Tool window will display the MCU’s pinout.  For your designs, the pinout will be based on the layout of the 32F411EDISCOVERY board.  This new project, DEMO, is listed under the Project Explorer tab where the  icon next to the project name indicates it is the current (active) project.

- With three exceptions, this design will use the default configuration.  The first two changes required are to disable USB_HOST and USB_OTG_FS.  To do so, in the Pinout & Configuration Window, select A-Z, USB_HOST, and Disable (from the dropdown window).  Repeat this for USB_OTG_FS.

The third changes required is to change the High Speed Clock (HSE) from BYPASS Clock Source to Crystal/Ceramic Resonator. To do so, in the Pinout & Configuration Window, select A-Z, RCC, and change the High-Speed Clock (HSE) to Crystal/Ceramic Resonator (from the dropdown window). After this change, you must power cycle the board.

- To save the hardware configuration and to create the source code files, select the X to close the Device Configuration Tool.

- Select “Remember my decision” and then select Yes.  (note: again, this window will only appear the first time you run the software).

- Within the Project Explorer tab, expand the Core folder then the Src folder.  Open the main.c file to display the source code that was generated by the Device Configuration Tool.  This file is over 400 lines long and contains the code required to initialize the hardware on the 32F411EDISCOVERY board based on the hardware option selected in the Device Configuration Tool.

- Shown below is the main() section of main.c. Note the highlighted comment blocks.  To ensure you do not modify any of the hardware configuration code, ONLY place your user code within these regions.   Also, if/when you re-run the Device Configuration Tool an updated version of main.c will be generated.  If you limit your code placement to the highlighted regions, the updated main.c will retain your code, otherwise your code will be overwritten.

- Modify the while loop within main.c (lines 108-119) to contain the following code:

- We will use three different delays in these labs.

- The for loop above is a simple delay that makes the CPU spend its time counting to some big number. It is often considered the worst type of delay but the fact that it does not use any interrupts or require an RTOS makes it useful later in lab.

- HAL_Delay(delay) is part of the hardware abstraction layer (HAL). The argument delay is the number milliseconds you want to wait. HAL_Delay uses interrupts and keeps the CPU busy, making it less desirable.

- osDelay(delay) is part of the CMSIS-RTOS Library. The argument delay is the number SysTicks (often 1 SysTick = 1 millisecond) you want to wait. This RTOS function will change the status of the task to blocked and allow context switching to other tasks.

- After saving the code, compile main.c (Project  Build All).  If there are any build errors make the necessary modifications, save, and re-build.

- To download the executable to the 32F411EDISCOVERY board, select Run  Debug (F11).  You may see the following dialog box.  If you do, select the STM32Cortex-M C/C++ Application and select Yes.

- In the Edit launch configuration properties window, use all the default settings and then select OK.  In the Confirm Perspective Switch window, select “Remember my decision” and then select No.  (note: again, this window will only appear the first time you run the software).

- After you received the “Download verified successfully” message in the console window, select Run  Resume (F8).  Your code should now be running on the board.

- Using the code segment shown below as a guide (note, you’re NOT writing this code), modify main.c to do the following:

  - Initialized the Red and Blue LEDs on and the Green and Orange LEDs off.

  - Within the while loop…

    - toggle both the Red and Green LEDs.

    - execute a software delay { for (int i=0; i < 300000; i++); }

    - poll the pushbutton switch.

      - output the value of the pushbutton to the Orange LED.

      - output the value of the !pushbutton to the Blue LED.